Calculation of  effective dose due to immersion in cloud of gamma emitters

1) Discussion and equation

The direct exposure pathway for -emitting radionuclides released in to the atmosphere will be external -dose to the whole body from the radioactive material in the plume. Estimates for external exposure to  radiation due to immersion in contaminated air provide conservative estimates for exposure to an overhead plume, due to the factor of distance. The values are derived under the assumption that a semi-infinite cloud correctly approximates the plume. The equation is:

Eimmersion= C x CFimmersion x t

Where

Eimmersion =	Effective dose from external exposure due to immersion in contaminated air [mSv].

C =	Average concentration of radionuclide in air [kBq/m3].

CFimmersion =	Conversion factor for radionuclide [(mSv/h)/(kBq/m3)].

t  =	Exposure duration [h].

2) Conversion factors

| Radionuclide | CFimmersion (mSv/h)/(kBq/m3) |
| --- | --- |
| Na-22 | 4.8E-04 |
| Na-24 | 1.0E-03 |
| K-40 | 3.4E-05 |
| K-42 | 6.3E-05 |
| Sc-46 | 4.4E-04 |
| Ti-44 | 2.8E-05 |
| V-48 | 6.3E-04 |
| Cr-51 | 6.7E-06 |
| Mn-54 | 1.9E-04 |
| Mn-56 | 4.1E-04 |
| Fe-59 | 2.6E-04 |
| Co-58 | 2.1E-04 |
| Co-60 | 5.6E-04 |
| Cu-64 | 4.1E-05 |
| Zn-65 | 1.3E-04 |
| Se-75 | 8.5E-05 |
| Kr-85m | 3.4E-05 |
| Kr-87 | 1.9E-04 |
| Kr-88 | 4.8E-04 |
| Kr-89 | 4.4E-04 |
| Rb-86 | 2.1E-05 |
| Rb-88 | 1.5E-04 |
| Rb-89 | 4.8E-04 |
| Sr-91 | 1.5E-04 |
| Zr-95 | 1.6E-04 |
| Zr-97 | 4.1E-05 |
| Nb-94 | 3.4E-04 |
| Nb-95 | 1.7E-04 |
| Mo-99 | 3.4E-05 |
| Tc-99m | 2.8E-05 |
| Ru-103 | 1.0E-04 |
| Ru-105 | 1.7E-04 |
| Ru/Rh-106a | 4.4E-05 |
| Ag-ll0m | 5.9E-04 |
| In-114m | 1.9E-05 |
| Sn-113 | 1.8E-06 |
| Sn-123 | 1.5E-06 |
| Sn-125 | 6.7E-05 |
| Sn-126 | 1.0E-05 |
| Sb-124 | 4.1E-04 |
| Sb-126 | 5.9E-04 |
| Sb-127 | 1.4E-04 |
| Sb-129 | 3.2E-04 |
| Te-129 | 1.1E-05 |
| Te-129m | 7.4E-06 |
| Te-131m | 3.1E-04 |
| Te-132 | 4.4E-05 |
| Te-134 | 1.9E-04 |
| I-125 | 2.3E-06 |
| I-129 | 1.8E-06 |
| I-131 | 8.1E-05 |
| I-132 | 5.2E-04 |
| I-133 | 1.3E-04 |
| I-134 | 5.9E-04 |
| I-135 | 3.5E-04 |
| Xe-131m | 1.8E-06 |
| Xe-133 | 7.4E-06 |
| Xe-133m | 6.3E-06 |
| Xe-135 | 5.2E-05 |
| Xe-135m | 9.3E-05 |
| Xe-137 | 4.1E-05 |
| Xe-138 | 2.6E-04 |
| Cs-134 | 3.4E-04 |
| Cs-136 | 4.8E-04 |
| Cs/Ba-137a | 1.3E-04 |
| Cs-138 | 5.2E-04 |
| Ba-133 | 7.8E-05 |
| Ba-139 | 7.8E-06 |
| Ba-140 | 4.1E-05 |
| La-140 | 5.2E-04 |
| La-141 | 9.3E-06 |

a	The contribution from the short-lived daughters is included in the factors for the parent radionuclide.

Radionuclides with a CFimmersion less than 10-6 [(mSv/h)/(kBq/m3)] have not been mentioned.

## 3) Example

What is the effective dose received by a person who remains 3 hours in a cloud of radionuclides containing an average of 27 kBq/m3 of Cs/Ba-137 and 45 kBq/m3 of Cs-134 ?

Cs/Ba-137 : CFimmersion = 1.3E-04 (mSv/h)/(kBq/m3).

Cs-134 : CFimmersion = 3.4E-04 (mSv/h)/(kBq/m3).

## Eimmersion= C x CFimmersion x t

Eimmersion  (Cs/Ba-137) =  0.01 mSv : Eimmersion (Cs-134) =  0.046 mSv.

Total effective dose from the cloud immersion in 3 hours = 0.06 mSv.

## 4) References

US ENVIRONMENTAL PROTECTION AGENCY, External exposure to Radionuclides in air, water and soil, Federal Guidance Report No 12, US EPA, EPA 402-R-93-081, Washington DC, May 1993.

IAEA TECDOC – 1162: Generic procedures for assessment and response during a radiological emergency. IAEA, August 2000.