Calculation of dose to skin due to beta emitters

- Discussion and equation

Skin dose is very difficult to measure directly and is usually estimated. The beta dose rate to the skin expressed in terms of average surface concentrations of a radionuclide on the skin gives more reliable estimates for this exposure pathway. However, the data in the literature vary as much as an order of magnitude. The dose calculated here is to the basal layer of the skin (70 m in depth) due to beta rays and electrons. The gamma contribution to the dose rate is generally a few per cent. The contamination is supposed to be uniformly and thinly spread over the skin. The equation is:

HT(skin) = Cskin x CFBeta−skin x tSFBeta

Where

HT(skin)	=	Equivalent dose to the skin [Gy].

Cskin	=	Average surface concentration of radionuclide on skin or clothing [Bq/cm2].

CFBeta-skin =	Conversion factor: skin beta dose rate [(Gy/h)/(Bq/cm2)].

SFBeta = Shielding factor for beta radiation due to clothing; representative values of shielding factors are approximately 3 - 5 for light clothing and 1000 for heavy clothing.

t = Time of exposure [h]

The equation should be calculated for each radionuclide present and the equivalent doses should be summed.

- Conversion factors

| Radionuclide | CFBeta-skin (µGy/h)/(Bq/cm2) |
| --- | --- |
| H-3 | 0 |
| C-14 | 0.32 |
| F-18 | 1.9 |
| Na-22 | 1.7 |
| Na-24 | 2.2 |
| Al-26 | 1.8 |
| P-32 | 1.9 |
| P-33 | 0.86 |
| S-35 | 0.35 |
| Cl-36 | 1.8 |
| K-40 | 1.5 |
| K-42 | 2.2 |
| K-43 | 1.9 |
| Ca-45 | 0.84 |
| Ca-47/Sc-47 | 3.5 |
| Sc-46 | 1.4 |
| Sc-47 | 1.5 |
| Cr-51 | 0.015 |
| Mn-52 | 0.761 |
| Mn-54 | 0.062 |
| Mn-56 | 2.4 |
| Fe-52 | 1.1 |
| Fe-55 | 0.016 |
| Fe-59 | 0.97 |
| Co-56 | 0.55 |
| Co-57 | 0.12 |
| Co-58 | 0.30 |
| Co-60 | 0.78 |
| Ni-63 | 0 |
| Ni-65 | 2.2 |
| Cu-64 | 1.0 |
| Cu-67 | 1.3 |
| Zn-65 | 0.076 |
| Ga-66 | 1.6 |
| Ga-67 | 0.35 |
| Ga-68 | 1.8 |
| As-76 | 2.1 |
| Se-75 | 0.14 |
| Br-77 | 0.01 |
| Br-82 | 1.5 |
| Rb-87 | 1.9 |
| Sr-85 | 0.06 |
| Sr-89 | 1.8 |
| Sr-90/Y-90 | 3.5 |
| Y-90 | 2.0 |
| Zr-95/Nb-95 | 1.6 |
| Mo-99/Tc-99m | 1.9 |
| Tc-99m | 0.25 |
| Tc-99 | 1.2 |
| Ru-103/ Rh-103m | 0.78 |
| Ru-106/ Rh-106 | 2.2 |
| Ag-110m | 0.68 |
| Ag-111 | 1.8 |
| Cd-109 | 0.54 |
| In-111 | 0.38 |
| In-113m | 0.73 |
| In-115m | 1.3 |
| Sn-125 | 2.3 |
| Sb-122 | 2.2 |
| Sb-124 | 2.2 |
| Sb-126 | 1.8 |
| Te-123m | 1.1 |
| Te-132 | 0.78 |
| I-123 | 0.38 |
| I-124 | 0.52 |
| I-125 | 0.021 |
| I-131 | 1.6 |
| Cs-131 | 0.01 |
| Cs-134 | 1.4 |
| Cs-137 | 1.6 |
| Ba-133 | 0.13 |
| Ba-140/La-140 | 3.8 |
| La-140 | 2.1 |
| Ce-139 | 0.49 |
| Ce-141 | 1.8 |
| Ce-143 | 2.0 |
| Pr-143 | 1.7 |
| Pm-147 | 0.6 |
| Sm-153 | 1.6 |
| Eu-152 | 0.92 |
| Eu-154 | 2.1 |
| Eu-156 | 1.2 |
| Er-169 | 1.1 |
| Yb-169 | 1.0 |
| Re-186 | 1.8 |
| Re-188 | 2.3 |
| Ir-192 | 1.9 |
| Au-198 | 1.7 |
| Hg-197 | 0.092 |
| Hg-203 | 0.89 |
| Tl-201 | 0.27 |
| Tl-204 | 1.6 |
| Pb-210 | 0.0084 |
| Po-210 | 6.90E-07 |
| U-235 | 0.18 |
| U-238 | 2.30E-03 |
| Pu-238 | 3.70E-03 |
| Pu-239 | 1.40E-03 |
| Am-241 | 0.019 |
| Cm-244 | 2.20E-03 |
| Cf-252 | 3.20E-03 |

## 3) Example

What is the equivalent tissue dose to the skin for a person who has an average skin contamination of 1250 Bq/cm2 of Mo-99/Tcm-99 and 250 Bq/cm2 of I-131 for 2 hours? The SFBeta is considered to be equal to 1 in this case.

HT(skin) = Cskin x CFBeta−skin x tSFBeta

Mo-99/Tcm-99: CFBeta-skin = 1.9 (µGy/h)/(Bq/cm2)

I - 131 : CFBeta-skin = 1.6 (µGy/h)/(Bq/cm2)

HT(skin)  (Mo-99/Tcm-99) = 4750 Gy : HT(skin) (I-131) = 800 Gy

Total equivalent dose to the skin from this contamination in 2 hours = 5.6 mGy.

## 4) References

DELACROIX, D., GUERRE, J.P., LEBLANC, P., HICKMAN, C., Radionuclide and Radiation Protection Data Handbook 1998, Radiation Protection Dosimetry, Vol. 76 Nos. 1-2 1998.