PHY 243 – Chapter 8 Homework Problems

- An open-air ionization chamber with a 300 cm3 volume is calibrated at a facility in Florida. The registered atmospheric pressure was 760 torr and the ambient temperature was 20C. The sample is taken to Denver, CO and used for survey work on a cool day. The outside temperature was 55F and the pressure read 0.75 atm. If the meter read 12 mR/hr, what was the true exposure rate at that location?

- Calculate the radiation exposure rate registered by an air-filled ionization chamber which registered a steady state current of 5 picoamperes (pA) while measuring a gamma ray field. The chamber volume was 150 cm3, the ambient temperature was 25C, and the pressure was 790 mm Hg (torr). (Note that the Roentgen is officially defined at 0C and 760 mm Hg).

- Using a benchtop Geiger counter, you obtain the following data using a “split source”. Calculate the detector resolving time.

  - S1: 101,980 counts/5 min.

  - S2:139,500 counts/5 min.

  - S1+S2:199,000 counts/5 min.

  - Background: 291 counts/min.

- Assuming nearly complete collection efficiency for photoelectrons in a typical PMT, if the multiplication factor () is 5, how much gain will one get from a tube with ten dynode stages? How many stages would be needed to get the same gain from “high-yield NEA” type dynodes which have a  of 55?

- Where will the Compton edge occur for a 1.46 MeV photon? Roughly sketch out what the gamma spectrum will look like for detection of a single photon at this energy, showing the photopeak, Compton gap, Compton edge, and Compton continuum.

- A laboratory standard has a mean activity of 256 counts/min. What is the probability of a 1 minute count yielding a value of exactly 256 counts? What is the probability of seeing any value between 240-272 counts in a 1 minute count?

- A sample is counted for 30 minutes, and yields 1220 counts. The background count rate for the detector is 5 cpm.

  - What are the standard deviation of the gross and background count rates for a 30 min count time?

  - What is the net count rate and its standard deviation?

  - Define a 95% confidence interval for the net count rate.

- A detector shows a background count of 500 counts in a 30 minute time. What would the LLD for this system be for counting times of:

  - 10 minutes

  - 30 minutes

  - 10 hours

- A detector system shows a background counting rate of 55 cpm. The detector efficiency for counting the radiation of interest is 35%. The chemical recovery for the process to prepare the samples is 80%. The radiation of interest appears with an abundance of 74% in the radionuclide’s decay scheme. Calculate the following quantities for a 1 minute count of a 3 kg sample on this detector system. Express all answers in Bq/kg (absolute).

  - LC

  - LD

  - LQ