Calculation of effective dose from exposure to ground contamination.

1) Equation

The effective dose calculated includes the external dose and committed dose from inhalation (resuspension) resulting from remaining on contaminated ground for the period of concern – first month, second month or lifetime (50 years). Once the radionuclide concentrations on the ground are known, the effective dose can be estimated.  The equation is:

Et= Cground x CFground,t

Where

Et	=	Effective dose [mSv] from deposition over the time interval t, where t = first month, second month or 50 years.

Cground	= Average ground concentration of radionuclide [kBq/m2].

CFground, t	= Conversion factor: effective dose per unit deposition for radionuclide for the specified time interval t = first month, second month or 50 years (mSv/kBq/m2). Includes external dose and committed effective dose from inhalation due to resuspension resulting from remaining on contaminated ground for the period of concern.

The equation should be calculated for each radionuclide present and the effective doses should be summed.

# 2) Conversion factors

| Radionuclide | Conversion factor (mSv/kBq/m2) |  |  |
| --- | --- | --- | --- |
|  | 1st month | 2nd month | 50 years |
| C-14 | 5.2E-07 | 4.9E-07 | 1.0E-04 |
| Na-22 | 3.7E-03 | 3.4E-03 | 8.4E-02 |
| Na-24 | 2.0E-04 | 0.0E+00 | 2.0E-04 |
| P-32 | 5.3E-06 | 1.2E-06 | 6.8E-06 |
| P-33 | 1.1E-06 | 4.4E-07 | 1.8E-06 |
| S-35 | 1.2E-06 | 8.7E-07 | 4.7E-06 |
| Cl-36 | 8.1E-06 | 7.7E-06 | 1.6E-03 |
| K-40 | 2.6E-04 | 2.5E-04 | 5.3E-02 |
| K-42 | 1.2E-05 | 0.0E+00 | 1.2E-05 |
| Ca-45 | 2.9E-06 | 2.4E-06 | 1.8E-05 |
| Sc-46 | 3.0E-03 | 2.2E-03 | 1.2E-02 |
| Ti-44+Sc-44 | 4.0E-03 | 3.8E-03 | 5.9E-01 |
| V-48 | 2.8E-03 | 7.1E-04 | 3.7E-03 |
| Cr-51 | 3.8E-05 | 1.7E-05 | 6.9E-05 |
| Mn-54 | 1.4E-03 | 1.2E-03 | 1.4E-02 |
| Mn-56 | 1.5E-05 | 0.0E+00 | 1.5E-05 |
| Fe-55 | 9.1E-07 | 8.5E-07 | 2.2E-05 |
| Co-58 | 1.6E-03 | 9.4E-04 | 3.9E-03 |
| Co-60 | 4.2E-03 | 3.9E-03 | 1.7E-01 |
| Ni-63 | 5.3E-07 | 5.0E-07 | 9.1E-05 |
| Cu-64 | 8.6E-06 | 0.0E+00 | 8.6E-06 |
| Zn-65 | 9.4E-04 | 8.2E-04 | 8.0E-03 |
| Ge-68+Ga-68 | 1.6E-03 | 1.4E-03 | 1.5E-02 |
| Se-75 | 6.2E-04 | 4.9E-04 | 3.1E-03 |
| Rb-86 | 1.0E-04 | 3.2E-05 | 1.5E-04 |
| Sr-89 | 1.1E-05 | 6.6E-06 | 2.8E-05 |
| Sr-90 | 1.7E-04 | 1.6E-04 | 2.1E-02 |
| Sr-91 | 3.4E-05 | 7.5E-08 | 3.4E-05 |
| Y-90 | 1.7E-06 | 6.7E-10 | 1.7E-06 |
| Y-91 | 1.7E-05 | 1.1E-05 | 4.9E-05 |
| Y-91m | 1.6E-06 | 6.5E-09 | 1.6E-06 |
| Zr-93 | 2.2E-05 | 2.1E-05 | 4.8E-03 |
| Zr-95 | 1.4E-03 | 1.3E-03 | 6.8E-03 |
| Nb-94 | 2.7E-03 | 2.6E-03 | 5.5E-01 |
| Nb-95 | 1.0E-03 | 5.2E-04 | 2.1E-03 |
| Mo-99+Tc-99m | 6.1E-05 | 3.1E-08 | 6.1E-05 |
| Tc-99 | 4.1E-06 | 3.9E-06 | 8.2E-04 |
| Tc-99m | 2.7E-06 | 1.2E-14 | 2.7E-06 |
| Ru-103 | 6.4E-04 | 3.6E-04 | 1.5E-03 |
| Ru-105 | 1.4E-05 | 1.8E-12 | 1.4E-05 |
| Ru-106+Rh-106 | 4.2E-04 | 3.8E-04 | 4.8E-03 |
| Ag-110m | 4.5E-03 | 3.9E-03 | 3.9E-02 |
| Cd-109+Ag-109m | 6.4E-05 | 5.8E-05 | 8.6E-04 |
| Cd-113m | 1.1E-04 | 1.1E-04 | 9.2E-03 |
| In-114m | 4.5E-04 | 3.5E-04 | 2.2E-03 |
| Sn-113+In-113m | 2.2E-05 | 1.7E-05 | 1.2E-04 |
| Sn-123 | 3.2E-03 | 3.2E-03 | 7.0E-01 |
| Sn-126+Sb-126m | 2.6E-03 | 1.7E-03 | 7.8E-03 |
| Sb-124 | 2.4E-03 | 4.2E-04 | 2.9E-03 |
| Sb-126m | 2.3E-04 | 1.1E-06 | 2.3E-04 |
| Sb-127 | 2.3E-05 | 4.9E-08 | 2.3E-05 |
| Sb-129 | 3.7E-06 | 3.6E-08 | 3.7E-06 |
| Te-127 | 1.8E-07 | 0.0E+00 | 1.8E-07 |
| Te-127m | 3.4E-05 | 2.7E-05 | 1.6E-04 |
| Te-129 | 2.5E-07 | 9.7E-16 | 2.5E-07 |
| Te-129m | 1.1E-04 | 5.4E-05 | 2.2E-04 |
| Te-131 | 1.2E-06 | 3.8E-08 | 1.2E-06 |
| Te-131m | 2.0E-04 | 3.3E-06 | 2.0E-04 |
| Te-132 | 6.9E-04 | 1.1E-06 | 6.9E-04 |
| I-125 | 7.8E-05 | 5.2E-05 | 2.4E-04 |
| I-129 | 1.7E-04 | 1.6E-04 | 3.4E-02 |
| I-131 | 2.5E-04 | 1.8E-05 | 2.7E-04 |
| I-132 | 1.9E-05 | 0.0E+00 | 1.9E-05 |
| I-133 | 4.5E-05 | 0.0E+00 | 4.5E-05 |
| I-134 | 8.1E-06 | 0.0E+00 | 8.1E-06 |
| I-135+Xe-135m | 3.7E-05 | 0.0E+00 | 3.7E-05 |
| Cs-134 | 2.7E-03 | 2.5E-03 | 5.1E-03 |
| Cs-135 | 7.0E-07 | 3.9E-07 | 8.5E-06 |
| Cs-136 | 1.9E-03 | 3.6E-04 | 2.3E-03 |
| Cs-137+Ba-137m | 9.9E-04 | 9.4E-04 | 1.3E-01 |
| Ba-133 | 7.0E-04 | 6.6E-04 | 4.8E-02 |
| Ba-140 | 2.0E-03 | 4.4E-03 | 2.5E-03 |
| La-140 | 3.2E-04 | 1.2E-09 | 3.2E-04 |
| Ce-141 | 9.9E-05 | 4.9E-05 | 2.0E-04 |
| Ce-144+Pr-144 | 1.5E-04 | 1.3E-04 | 1.4E-03 |
| Pr-144 | 4.0E-08 | 0.0E+00 | 4.0E-08 |
| Pr-144m | 2.2E-08 | 0.0E+00 | 2.2E-08 |
| Pm-145 | 6.0E-05 | 5.7E-05 | 5.8E-03 |
| Pm-147 | 4.4E-06 | 4.1E-06 | 1.0E-04 |
| Sm-151 | 3.5E-06 | 3.3E-06 | 5.9E-04 |
| Eu-152 | 2.0E-03 | 1.9E-03 | 1.6E-01 |
| Eu-154 | 2.1E-03 | 2.0E-03 | 1.3E-01 |
| Eu-155 | 1.1E-04 | 1.0E-04 | 4.2E-03 |
| Gd-153 | 1.8E-04 | 1.6E-04 | 1.5E-03 |
| Tb-160 | 1.7E-03 | 1.2E-03 | 5.8E-03 |
| Ho-166m | 3.1E-03 | 2.9E-03 | 6.1E-01 |
| Tm-170 | 1.6E-05 | 1.3E-05 | 8.5E-05 |
| Yb-169 | 4.0E-04 | 2.0E-04 | 7.9E-04 |
| Hf-181 | 7.7E-04 | 4.5E-04 | 1.8E-03 |
| Ta-182 | 2.0E-03 | 1.6E-03 | 9.7E-03 |
| W-187 | 4.1E-05 | 0.0E+00 | 4.1E-05 |
| Ir-192 | 1.2E-03 | 8.9E-04 | 4.4E-03 |
| Au-198 | 9.4E-05 | 3.9E-08 | 9.4E-05 |
| Hg-203 | 3.3E-04 | 2.0E-04 | 8.5E-04 |
| Tl-204 | 4.0E-06 | 3.8E-06 | 1.2E-04 |
| Pb-210 | 1.9E-03 | 2.2E-03 | 5.9E-01 |
| Bi-207 | 2.6E-03 | 2.5E-03 | 3.4E-01 |
| Bi-210 | 1.2E-04 | 1.1E-04 | 7.3E-04 |
| Po-210 | 3.5E-03 | 2.9E-03 | 2.0E-02 |
| Ra-226 | 9.2E-03 | 9.2E-03 | 1.9E+00 |
| Ac-227 | 4.6E-01 | 4.4E-01 | 5.1E+01 |
| Ac-228 | 3.6E-05 | 1.4E-05 | 3.0E-04 |
| Th-227 | 7.7E-03 | 3.7E-03 | 1.3E-02 |
| Th-228 | 4.2E-02 | 3.9E-02 | 7.7E-01 |
| Th-230 | 3.7E-02 | 3.5E-02 | 7.5E+00 |
| Th-232 | 1.9E-01 | 1.8E-01 | 4.6E+01 |
| Pa-231 | 1.2E-01 | 1.1E-01 | 6.7E+01 |
| U-232 | 3.2E-02 | 3.1E-02 | 1.2E+01 |
| U-233 | 8.0E-03 | 7.6E-03 | 1.7E+00 |
| U-234 | 7.9E-03 | 7.4E-03 | 1.6E+00 |
| U-235 | 7.4E-03 | 7.0E-03 | 1.5E+00 |
| U-236 | 7.3E-03 | 6.9E-03 | 1.5E+00 |
| U-238 | 6.8E-03 | 6.4E-03 | 1.4E+00 |
| U Dep & Natural | 6.8E-03 | 6.4E-03 | 1.4E+00 |
| U Enriched | 7.9E-03 | 7.4E-03 | 1.6E+00 |
| UF6g (U234) | 7.9E-03 | 7.4E-03 | 1.6E+00 |
| Np-237 | 2.6E-02 | 2.5E-02 | 5.3E+00 |
| Np-239 | 3.4E-05 | 6.4E-09 | 3.4E-05 |
| Pu-236 | 1.6E-02 | 1.5E-02 | 8.0E-01 |
| Pu-238 | 3.9E-02 | 3.7E-02 | 6.6E+00 |
| Pu-239 | 4.2E-02 | 4.0E-02 | 8.5E+00 |
| Pu-240 | 4.2E-02 | 4.0E-02 | 8.4E+00 |
| Pu-241 | 7.6E-04 | 7.2E-04 | 1.9E-01 |
| Pu-242 | 4.0E-02 | 3.8E-02 | 8.0E+00 |
| Am-241 | 3.5E-02 | 3.3E-02 | 6.7E+00 |
| Am-242m | 3.2E-02 | 3.0E-02 | 6.3E+00 |
| Am-243 | 3.5E-02 | 3.3E-02 | 7.0E+00 |
| Cm-242 | 4.2E-03 | 3.5E-03 | 5.9E-02 |
| Cm-243 | 3.5E-02 | 3.3E-02 | 4.3E+00 |
| Cm-244 | 2.9E-02 | 2.7E-02 | 2.8E+00 |
| Cm-245 | 5.0E-02 | 4.7E-02 | 1.0E+01 |
| Cf-252 | 1.7E-02 | 1.5E-02 | 3.9E-01 |

## 3) Example

What is the effective dose received by a person who remains for the first 1 month on ground contaminated with an average of 250 Bq/m2 of Pu-239 and 1100 Bq/m2 of Am-241?

Pu-239: CFground, t = 4.2E-02 (mSv/kBq/m2).

Am-241: CFground,t = 3.5E-02 (mSv/kBq/m2).

Et= Cground x CFground,t

Et (Pu-239) = 0.0105 mSv, Et (Am-241) = 0.0385 mSv

Total effective dose in 1 month for this contamination = 0.049 mSv.

## 4) References

US ENVIRONMENTAL PROTECTION AGENCY, External Exposure to Radionuclides in Air, Water, and Soil, Federal Guidance Report No. 12, US EPA, EPA-402-R-93-081, Washington DC, September 1993.

US ENVIRONMENTAL PROTECTION AGENCY, Manual of Protective Action Guides and Protective Actions for Nuclear Incidents, US EPA, EPA 400-R-92-001, Washington DC, May 1992.

INTERNATIONAL ATOMIC ENERGY AGENCY, Derived Intervention Levels for Application in Controlling Radiation Doses to the Public in the event of a Nuclear Accident or Radiological Emergency, Safety Series No. 81, IAEA, Vienna (1986).

INTERNATIONAL ATOMIC ENERGY AGENCY, Generic Procedures for Assessment and Response during an Radiological Emergency, IAEA TECDOC- 1162, IAEA, Vienna (2000).